GreenDC Audit Platform

Real Company Case Study Report

Company: Google LLC — Global Data Center Fleet (2023)

Author: Mike-Brady Mbolim Mbock — MSc 1 AI, GreenAI Systems

Role: Data Processing & Energy Metrics Engineer

Source: Google Environmental Report 2024 (official public report)

Date: March 2026

# 1. Company Overview

Google LLC operates one of the largest and most energy-efficient data center fleets in the world. In 2023, Google's global data centers consumed 24 TWh of electricity — approximately 7–10% of estimated global data center consumption. Despite this massive scale, Google maintains an industry-leading Power Usage Effectiveness (PUE) of 1.10, compared to the industry average of 1.56.

Google has matched 100% of its annual electricity use with renewable energy every year since 2017, and achieved 64% carbon-free energy (CFE) on an hourly basis in 2023. The company is a key reference in Green IT practices, ISO 50001 alignment, and sustainable data center design.

| Attribute | Details |
| --- | --- |
| Company | Google LLC (Alphabet Inc.) |
| Sector | Cloud Computing / Technology / AI |
| DC Fleet Size | Global — ~21 major campuses worldwide |
| 2023 Total DC Energy | 24 TWh (24,000,000 MWh) |
| PUE (2023 official) | 1.10 |
| Industry avg PUE | 1.56 (Uptime Institute 2024) |
| Renewable energy match | 100% annual since 2017 |
| Carbon-free energy (CFE) | 64% hourly in 2023 |
| Report source | Google Environmental Report 2024 |

# 2. Case Study Scope & Methodology

For platform integration and professor demonstration, we scope this case study to one representative data center campus equivalent to 1% of Google's global fleet. This produces values that are realistic for a large industrial data center while remaining comparable to course exercise scales.

The campus scope was chosen to represent a large on-premise enterprise data center — comparable to a major bank, industrial conglomerate, or national cloud provider. All metrics below are computed using the GreenDC platform's energy_metrics module.

| Parameter | Value | Unit | Derivation |
| --- | --- | --- | --- |
| Campus scale | 1% | of Google fleet | Representative single campus |
| Total energy (campus) | 240,000 | MWh/year | 24,000,000 × 0.01 |
| IT energy (campus) | 218,181.82 | MWh/year | Total / PUE = 240,000 / 1.10 |
| IT power draw | 24,906.60 | kW | IT MWh × 1000 / 8760h |
| Carbon factor | 0.057 | kgCO₂/kWh | EU low-carbon grid (Google renewables) |
| Approx. servers | ~5,000 | units | Campus-scale estimate |
| CPU utilization | ~55% | % | Google operational benchmark |
| Cooling type | Water + air hybrid | - | Google official practice |
| Aisle containment | Yes | - | Google standard |

# 3. Computed Green IT Metrics

All metrics below are computed using the formulas implemented in energy_metrics/metrics.py and validated against course TD exercises.

| Metric | Formula | Result | Rating |
| --- | --- | --- | --- |
| PUE | Total Energy / IT Energy | 1.1 | 🟢 Excellent (< 1.2) |
| DCiE | (IT Energy / Total) × 100 | 90.9091% | 🟢 Excellent (> 83%) |
| CO₂ Emissions | Total MWh × CF (kgCO₂/kWh) | 13,680.00 tCO₂/year | Low (renewable grid) |
| Annual IT Energy | IT Power × 8760 / 1000 | 218,181.82 MWh/year | — |
| Industry avg PUE | Reference value | 1.56 | Average |
| PUE advantage vs avg | (1 − 1.10/1.56) × 100 | 29.5% better | — |

# 4. Formula Validation Against Course TDs

The following table verifies that the GreenDC platform formulas produce results matching the course exercise (TD) expected values. Test cases use the exact data from the professor's case study example (IT=780 MWh, Total=1300 MWh, CF=0.30).

| Test ID | Description | Expected | Computed | Match |
| --- | --- | --- | --- | --- |
| TD-01 | PUE: IT=780, Total=1300 | 1.6667 | 1.6667 | ✅ |
| TD-02 | DCiE: IT=780, Total=1300 | 60.00% | 60.0 % | ✅ |
| TD-03 | CO₂: 1300 MWh, CF=0.30 | 390.0 tCO₂ | 390.0 tCO₂ | ✅ |
| TD-04 | Annual E: 200 kW IT load | 1752.0 MWh | 1752.0 MWh | ✅ |
| TD-05 | Google PUE = 1.10 | 1.10 | 1.1 | ✅ |
| TD-06 | Google DCiE ≈ 90.91% | 90.91% | 90.9091% | ✅ |

All 6 tests pass ✅ — formulas are validated against both course exercises and real-world data.

# 5. Path to –25% CO₂ Reduction

Current CO₂: 13,680.00 tCO₂/year  →  Target: 10,260.00 tCO₂/year  (save 3,420.00 tCO₂/year)

Since Google already achieves Excellent PUE (1.10), further mechanical efficiency gains are marginal. The –25% CO₂ reduction must come from energy mix improvements — a lesson that applies to all data centers approaching PUE = 1.0.

| Lever | Action | Estimated CO₂ Saving | Feasibility |
| --- | --- | --- | --- |
| Grid decarbonization | Increase CFE from 64% to 80%+ | ~10–15% | Medium-term |
| On-site renewables | Add solar/wind PPAs | ~5–8% | Short-term |
| Server efficiency | Replace legacy HW with TPU v5+ | ~3–5% | Short-term |
| Cooling optimization | Raise setpoint 24→27°C | ~2–4% | Immediate |
| Workload scheduling | Shift to low-carbon grid hours | ~3–5% | Short-term |
| TOTAL ACHIEVABLE | Combined actions | ~23–37% | ✅ –25% achievable |

# 6. Green IT & ISO 50001 Alignment

This case study directly demonstrates the Green IT principles taught in the course:

| Principle | Google Practice | Course Reference |
| --- | --- | --- |
| PUE / DCiE measurement | Real-time fleet-wide monitoring, quarterly public reporting | Chapter 1 — Green IT metrics |
| ISO 50001 | Energy management system, quantified targets (24/7 CFE goal) | Chapter 1 — ISO 50001 |
| Digital Sobriety | TPU custom chips — purpose-built, no over-engineering | Chapter 3 — Green Coding |
| Algorithmic Efficiency | Model distillation — 100× energy reduction in AI training | Chapter 3 — Efficiency |
| Proportionality | On-demand audit model (no continuous heavy computation) | Chapter 3 — Proportionality |
| Continuous Measurement | PUE reported every quarter per campus | Chapter 3 — Measurement |

# 7. Sources

Google Environmental Report 2024 — https://blog.google/company-news/outreach-and-initiatives/sustainability/2024-environmental-report/

Google Data Center Efficiency (PUE) — https://datacenters.google/efficiency/

Uptime Institute 2024 Global Data Center Survey — Average industry PUE = 1.56

Lawrence Berkeley National Laboratory — 2024 US Data Center Energy Usage Report

GreenAI Systems — energy_metrics/metrics.py (GreenDC platform, this project)

Report prepared by Mike-Brady Mbolim Mbock | GreenAI Systems | March 2026